DRAFT RFQ

1. Application

The nitrogen generator shall be used for providing a continuous nitrogen atmosphere to a Sealed Quench Heat Treatment Furnace.

2. Operating Requirements

| Description | Requirement |
| --- | --- |
| Nitrogen Consumption | 28 Nm³/hour (continuous) |
| Recommended Generator Capacity | Minimum 35 Nm³/hour |
| Plant Operation | 24 Hours/Day |
| Operating Days | 15 Days/Month |
| Operating Months | 12 Months/Year |
| Application | Sealed Quench Furnace |

3. Scope of Supply

- PSA Nitrogen Generator

- Air Compressor

- Air Receiver Tank

- Refrigerated or Desiccant Air Dryer

- Air Filters (Pre-filter, Coalescing Filter and Activated Carbon Filter)

- Nitrogen Buffer Tank

- Oxygen Analyzer

- Pressure Regulators

- Automatic PLC Control Panel

- Interconnecting piping and valves

- Instrumentation

- Electrical control system

- Safety devices

- Installation materials

- Testing and commissioning

- Operator training

4. Technical Requirements

- Nitrogen generation capacity (Nm³/hr)

- Nitrogen purity (99 %)

- Outlet pressure

- Dew point

- Electrical power requirement (kW)

- Air consumption

- Compressor specifications

- Overall dimensions

- Noise level

- Utility requirements

- Instrumentation provided

- Automation features

- Safety features

5. Commercial Information Required

- Equipment price

- Installation and commissioning charges

- Freight charges

- Delivery period

- Payment terms

- Recommended spare parts

- Preventive maintenance schedule

- Availability of local service support

- Country of origin

6. Documentation

- Technical brochure

- Product catalogue

- General arrangement drawing

- Process Flow Diagram (PFD)

- Piping & Instrumentation Diagram (P&ID), if available

- Electrical load list

- Foundation requirements

- Utility requirements

- List of major components

- Quality certifications (ISO, CE, etc.)

- Reference list of similar installations

7. Performance Guarantee

- Guaranteed nitrogen flow rate.

- Guaranteed nitrogen purity.

- Power consumption.

- Performance guarantee after commissioning.

8. After-Sales Support

- Local service support.

- Availability of spare parts.

- Response time for service calls.

9. Warranty

- PSA Nitrogen Generator

- Air Compressor

- Control System

- Instruments

- Complete System